OPC is open connectivity in industrial automation and the enterprise systems that support industry. OPC technology is managed by OPC foundation (http://www.opcfoundation.org) who adapts and creates specifications that fill industry-specific needs. Now OPC is a series of standards specifications. Nowadays more than 300 companies take are participants of OPC Foundation. Microsoft, GE, Siemens, ABB, Ford, GM, SAP AG and more are members that actively contribute and consume OPC technology.

OPCCheck utility requests value of specified item from OPC server. This command line utility was written according to OPC DA (OPC Data Access) specification that is based on COM/DCOM Microsoft's technologies.

OPCCheck can be used with HostMonitor by “Shell Script” test method.

Format of the command line:

> opccheck.exe -serverid:<Server_Prog_ID> -itemname:<item_name> [-host:<host_name>]

[-user:<username>] [-password:<password>] [-cache] [-verbose] [-AcceptUncertain]

[-greaterthan:<threshold> | -lessthan:<threshold> | -equalto:<threshold>

| -notequalto:<threshold> | -contains:<text> | -notcontain:<text>]

Parameters:

| -serverid | OPC Server ID, e.g. "Matrikon.OPC.Simulation.1". This parameter is obligatory. |
| --- | --- |
| -itemname | Item name to check. This is 2nd obligatory parameter. |
| Other parameters are optional |  |
| -host | Hostname or IP address of target OPC server. If parameter is omitted, local system will be requested |
| -user | Username for authentication |
| -password | Password for authentication |
| -verbose | Display additional information about received OPC packet |
| -cache | Tells utility to read data from OPC server cache. By default utility requests data directly from device |
| -AcceptUncertain | Tells utility to treat "Uncertain" quality as "Good" |
| The following optional parameters tell utility to compare value of the item to specified data and return “Bad” status when value fits specified condition |  |
| -greaterthan | Sets Bad status if counter value is greater than the threshold |
| -lessthan | Sets Bad status if counter value is less than threshold |
| -equalto | Sets Bad status if counter value is equal to specified threshold |
| -notequalto | Sets Bad status if counter value is not equal to the threshold |
| -contains | Sets Bad status if counter value contains specified text |
| -notcontain | Sets Bad status if counter value does not contain specified text |

Examples:

>opccheck -serverID:Matrikon.OPC.Simulation.1 -itemName:"Bucket Brigade.Real4" -greaterthan:120

>opccheck -serverID:Matrikon.OPC.Perfmon.1  -itemName:"Triangle Waves.Real8" -host:\\server.corp.com -cache

>opccheck -serverID:MRD.DA2.1 -itemName:"Bucket Brigade.Real4" -host:192.168.1.100 -AcceptUncertain -equalto:50

Utility provides the results of testing in 3 ways:

  - It displays information on the screen.

  - Utility returns exit code that could be checked by a parent application, thus you may use opccheck utility for “External” test method. 
Exit codes have the following meanings:

0:  Utility was able to retrieve data from OPC server. If “bad” threshold was specified, retrieved value does not fit the threshold

1:  Utility was able to retrieve data from OPC server. Retrieved value fits specified “bad” threshold (threshold can be specified by parameters like –greaterthan, -lessthan, etc)

2:  Some error occurred (e.g. invalid command line arguments, wrong item name, unavailable OPC server, etc).

  - Also utility returns special string that can be interpreted by HostMonitor when you setup “Shell Script” test method to launch opccheck.exe.

Utility may return the following statuses: Ok, Bad, Unknown

| Status field | Reply field shows | Meaning |
| --- | --- | --- |
| Ok | Value of the item | Reply field displays value of the item retrieved from OPC server. If “bad” threshold was specified, this value does not fit the threshold (if option such as –greaterthan, -lessthan was used) |
| Bad | Value of the item | Reply field displays item value retrieved from OPC server. This value fits specified “bad” threshold (threshold specified by parameters like –greaterthan, -lessthan, etc) |
| Unknown | Error message | Some error occurred (e.g. invalid command line arguments, wrong item name, unavailable OPC server) |

Minimum system requirements

- Windows 2000

- OPC COM ProxyStub Merge Module. This module available for download at http://www.opcfoundation.org/Downloads.aspx?CM=1&CN=KEY&CI=286&CU=18

(basically you need to install just 2 files: opccomn_ps.dll and opcproxy.dll.

E.g.

regsvr32 opccomn_ps.dll

regsvr32 opcproxy.dll)